Table Of Contents

1	Description of Manufacturing Process and Process Controls	3

1.1	Description of the Manufacturing Process and Facility	3

1.1.1	Building and facilities	3

1.1.1.1	Introduction	3

1.1.1.2	Nature of Construction and finishes	3

1.1.1.3	Heating, Ventilation and Air Conditioning (HVAC) systems	4

1.1.1.4	Water for Injection Generation, Storage and Distribution System	4

1.1.1.5	Nitrogen System	4

1.1.1.6	Manufacturing Locations	4

1.1.1.7	Equipment Location	5

1.1.2	Drug Product Manufacturing Process	5

1.1.2.1	Dispensing	6

1.1.2.2	Vial Loading and Transfer	7

1.1.2.3	Clean In Place (CIP) & Sterilization In Place (SIP) Details	7

1.1.2.4	Preparation for Manufacturing	7

1.1.2.5	Manufacturing Process	8

1.1.2.6	First Filtration Process	10

1.1.2.7	Final Filtration and Filling Process	11

1.1.2.8	Vial Collection	14

1.1.2.9	Final Filtration, Vial Filling and Half Stoppering	14

1.1.2.10	Lyophilization	15

1.1.2.11	LYO Loading Activity	16

1.1.2.12	LYO Unloading Activity	16

1.1.2.13	Capping Activity	17

1.1.2.14	Vial Collection & Automatic Leak test	17

1.1.2.15	Visual Inspection	Error! Bookmark not defined.

1.1.2.16	Terminal Sterilization	18

1.1.2.17	Visual Inspection	18

1.1.2.18	Labeling and Packaging	19

1.1.3	Stage-wise Proposed Yield	20

1.1.4	Holding Period Specifications	20

1.2	Manufacturing Process Flow Chart	21

1.3	Master Batch Manufacturing and Packaging Records	21

1.4	Sterilization/Depyrogenation	21

1.4.1	Terminal Sterilization (moist heat)	22

1.4.1.1	Terminal Sterilization Process and Critical Parameters	22

1.4.1.2	Methods and Controls to Monitor Production Cycles	23

1.4.1.3	Steam Sterilizer Load Patterns	23

1.4.1.4	Requalification or Revalidation	24

1.4.2	Component Sterilization/ Depyrogenation	26

1.4.2.1	Sterilization Process and Critical Parameters	26

1.4.2.2	Methods and Controls to Monitor Production Cycles	27

1.4.2.3	Autoclave Load Patterns	27

1.4.2.4	Requalification or Revalidation	27

1.4.3	Equipment Sterilization (Lyophilizer)	31

1.4.3.1	Sterilization Process and Critical Parameters of Lyophilizer	31

1.4.3.2	Methods and Controls to Monitor Production Cycles	32

1.4.3.3	Lyophlizer Load Patterns	32

1.4.3.4	Requalification or Revalidation	32

1.4.4	Environmental Monitoring	32

1.4.4.1	Microbial Monitoring	32

1.5	Reprocessing Statement	33

List of In-Text Tables

Table 1. Manufacturing Location	4

Table 2. Equipment Location	5

Table 3. Executed Vs. Proposed Commercial Batch Size Overview	21

Table 4. Proposed Commercial Batch Record Reference	21

Table 5. Steam Sterilizer Details	22

Table 6. Steam Sterilizer Cycle Parameters (Note: Parameters are changed based on equipment and model)	22

Table 7. Steam Sterilizer Load Details	23

Table 8. PP Vials (Minimum Load): Terminal Sterilization Load Details	24

Table 9. PP Vials (Maximum Load): Terminal Sterilization Load Details	24

Table 10. Autoclave Details	26

Table 11. Autoclave Cycle Parameters <Note: Parameters is changed based on equipment and model>	26

Table 12. Autoclave Load Details	27

Table 13. Lyophilizer Details	31

Table 14. Lyophilizer SIP Cycle Parameters	31

# Description of Manufacturing Process and Process Controls

## Description of the Manufacturing Process and Facility

### Building and facilities

#### Introduction

Company Name and Address

<Include the facility introduction from the Site Master File (SMF) and update it as per the current version. Also, specify the production line of the facility where the proposed product is manufactured.>

#### Nature of Construction and finishes

<Provide an introduction about the building, including its construction and finishes for the following areas, as per the current Site Master File (SMF)>

- Production Area:

- Warehouse Area:

- Microbiology Area:

- Laboratory Area:

- Listed below are facility layouts:

- Site Plan

- Architecture and Area classification Zooning Layout

- Layout of General Formulation - Injectable Manufacturing Floor

- Man and Material Movement

- Pressure Zoning Layout

- HVAC Zoning Layout

- Diagram for WFI storage & Distribution System

<Note: List the facility layouts involved in the manufacturing of the proposed product. Cross-verify with the BMR to confirm which specific areas are utilized during manufacturing. Please arrange high-resolution layout copies for better legibility>

#### Heating, Ventilation and Air Conditioning (HVAC) systems

<Information on the Heating, Ventilation, and Air Conditioning (HVAC) systems from the current Site Master File (SMF) needs to be included in this section>

<Add details regarding the sterile area cleanroom classification and the frequency of area qualification, as per the current Site Master File (SMF).>

#### Water for Injection Generation, Storage and Distribution System

<Include information on the above topic from the current Site Master File (SMF).>

#### Nitrogen System

<Include information on the above topic from the current Site Master File (SMF).>

#### Manufacturing Locations

The drug product will be manufactured on the <Add line name> of <manufacturing facility name and address>. The details regarding Area, Room No. and Classification for major manufacturing operations are tabulated below:

Table 1. Manufacturing Location

| Sr. No. | Operation | Area | Room No. | Classification |
| --- | --- | --- | --- | --- |
| <Stage Name> |  |  |  |  |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| <Stage Name> |  |  |  |  |
| 1 |  |  |  |  |
| 2 |  |  |  |  |

<Please include the manufacturing locations as mentioned in the final commercial batch manufacturing and packaging records.>

#### Equipment Location

Comparative details regarding the make, model and location within facility for the major equipment between the exhibit and proposed commercial batches is tabulated below:

Table 2. Equipment Location

| Sr. No. | Equipment name | Equipment Details |  | Location (Room No.) |
| --- | --- | --- | --- | --- |
|  |  | Exhibit | Commercial |  |
| <Stage Name> |  |  |  |  |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| <Stage Name> |  |  |  |  |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |

<Add the equipment details corresponding to those specified in the exhibit and commercial Batch Manufacturing Record (BMR) and Batch Packaging Record (BPR).>

- The compliance certificates (SS 316L/ polymeric compliance certificates) of all the manufacturing (equipment, Filter, Silicon tubes, Y connectors, etc.) used in exhibit batches and proposed for commercial, that directly come in contact with drug product are provided in Module 3.2.P.3.3 i.e. 32p33-compliance-certificates.

<Please arrange the compliance certificate (316 L for SS components and food grade for polymeric components which may contact drug product during manufacturing process from its specific vendors>

### Drug Product Manufacturing Process

<Add Product Name> in <add container closure information> was subjected to terminal sterilization suitability study based on F0 value derived from the product specified D-value study, wherein it was observed that the drug product remained stable without any signs of degradation. Thus, based on suitability study and adopting worst-case approach, a terminal sterilization cycle of F0 (Observed value) of NLT XX minutes is proposed for the drug product.

<The above is an example of terminal sterilization>

<Add a description if the product is manufactured using an aseptic process, as given below>

<Add Product Name>, is manufactured using an Aseptic sterilization process. As the product is sensitive to terminal steam Sterilization, the Aseptic Filtration process was selected as a suitable method for manufacturing.

A detailed summary of the drug product manufacturing is as follows:

<Include the manufacturing procedure derived from the commercial Batch Manufacturing Record (BMR) and Batch Packaging Record (BPR), along with the corresponding in-process control parameters. The information should be consistent with Section 3.2.P.3.4.>

<An example of the manufacturing procedures is provided below.>

#### Dispensing

i) Active Pharmaceutical Ingredient (API):

- APIs – API name are weighed and dispensed as per requirement.

Note: API manufacturing instruction note.

ii) Excipients:

- Excipient is weighed and dispensed as per requirement.

<Mention the Excipients list.>

iii) Primary Packaging Material (PPM):

• Primary packaging material is dispensed as per requirement.

| Packaging Configuration | Component | Description |
| --- | --- | --- |
| Packaging Material |  |  |

<Note: Add Primary packaging material section, if any intermediate packaging is there.>

iv). Processing Aids:

- Processing aids are dispensed as per requirement.

- Filters

< Include information about the filters used in both exhibit and commercial batches.>

<The description and catalogue numbers of the filters used in the exhibit and commercial batches should be added in this section.>

- Processing tubes

<Provide the name of the tubing used for filing, filtration, and transfer of the bulk solution, along with its dimensions and intended purpose>

#### Vial Loading and Transfer

- Perform the line clearance activity and record the differential pressure of [Vial Decartoning Area] prior to transfer of vials to [Next Area Name].

- Record the total good vials received, vials rejected during transfer, and total good vials transferred to [Vial Washing and DPT (Depyrogenation Tunnel) Room].

- Perform the line clearance activity and record the temperature and differential pressure of [Vial Washing Room] prior to vial loading.

- Record the good vials received, vials rejected during loading, leftover quantity of intact vials, and total good vials transferred to [Filling Area].

#### Clean In Place (CIP) & Sterilization In Place (SIP) Details

<Add information about the CIP and SIP from the Manufacturing Record (MR) and as per the Company SOP for each vessel used in the manufacturing process (if sterilization is an aseptic process).>

<If sterilization is a terminal procedure, then include only the CIP and SIP details of critical vessels.>

<Also add the SIP acceptance criteria, which should be part of the BMR and considered as an in-process control for the manufacturing process.>

#### Preparation for Manufacturing

<Add information from the Batch Manufacturing Record (BMR)>

<The process mentioned below is an example and should be modified as per the respective product.>

- Wash and dry the supporting accessories like SS container, measuring cylinder, glass beaker, processing tubing, glass bottle and pressure vessels, etc. for manufacturing.

- Perform the pre-integrity of final filter (for final filter details, please refer Point iv of Section 1.1.2.1), as applicable for commercial batches of respective fill volumes, using WFI as wetting medium (Limit: NLT XXX mbar at Temperature).

- After Completion of filter integrity, perform the drying of filter using 0.2μ filtered compressed air for NLT XX minutes and record the Applied Pressure (XXX bar).

- Record the differential pressure of respective LAF prior to unloading the Disinfectant room/ utensil washer as well as load preparation (Wrapping).

- Perform cleaning of stoppering vibrator and bowl, capping vibrator and bowl and record the detail.

- Transfer the XX mm rubber stoppers and Filling Load (forceps, needles, etc.) to the autoclave loading area.

- Load the components in steam sterilizer and perform sterilization [Temperature: XX°C – XX°C; Pressure: XX bar to XX bar; F0 value: NLT XX minutes].

- If Capsule filter is to be used, transfer the Capsule Filter Assembly to the autoclave loading area, load the capsule filter assembly in steam sterilizer and perform sterilization as per validated load pattern [Temperature: XX°C – XX°C; Pressure: XX bar to XX bar; F0 value: NLT XX minutes].

#### Manufacturing Process

- Check and ensure that the equipment, tubing, manufacturing accessories are clean.

- Record the differential pressure, % RH and temperature of the manufacturing room.

- Open the valve of filtered nitrogen to blanket the mixing vessel and purge the 0.2μ filtered Nitrogen continuously for at least XX minutes and record the pressure of applied nitrogen.

- Start the chilled water circulation in jacket of mixing vessel using Fill Jacket control. Fill the jacket and when load cell display reading stabilizes, tare the weight of mixing vessel to zero. Record displayed tare weight of mixing vessel.

<Note: Please add the capacity of each equipment and the limits for each in-process control in the manufacturing process. Ensure that this information matches the details provided in Section 3.2.P.3.4>

<The process mentioned below is an example.>

- Collect [Water for Injection (WFI)] in quantity approximately 10% excess to the batch size in [Mixing Vessel, Capacity XX L] using [WFI collection control system] under IPC observation. Record the quantity collected and temperature of WFI.

- Initiate chilled water circulation through the jacket with continuous stirring and nitrogen purging. Record stirrer speed [Limit: XXX–XXX RPM], cooling time, WFI temperature [Limit: 20–30°C], and pH [Limit: 5.0–7.0 at 20–25°C].

- Transfer cooled WFI from [Mixing Vessel] to [Pressure Vessel/Mixing Vessel] through [Silicone Tube Connection] by applying nitrogen pressure. Maintain a residual quantity of WFI in the mixing vessel as per process requirements. Record the transfer quantity, remaining WFI, and temperature.

- Add the dispensed quantity of [Ingredient-1] into the mixing vessel with continuous stirring at [20–30°C].

- Rinse the [Ingredient Container/Bag] with previously collected WFI to ensure complete transfer. Record WFI quantity used for rinsing and solution weight.

- Continue stirring until a clear, colorless solution is obtained. Record stirring speed [Limit: XXX–XXX RPM], mixing time [Limit: XX–XX minutes], solution temperature [Limit: 20–30°C], and bulk description [Clear, Colorless Solution].

- Add the dispensed quantity of [Ingredient-2] under continuous stirring at [20–30°C] and repeat rinsing for complete transfer. Record the relevant parameters.

- Drain the chilled water jacket and connect steam supply for heating. Record initial temperature before heating.

- Heat the bulk to approximately XX°C, for not less than XX minutes with continuous stirring until a clear, colorless solution is formed. Record stirrer speed, mixing time, temperature, and solution description.

- Ensure all vents of the vessel are closed to prevent evaporation loss during heating.

- Cool the bulk solution to [20–30°C] using jacket cooling with stirring. Record stirring speed, cooling time, temperature, and appearance of the bulk solution.

- Calculate and perform final WFI make-up to achieve target batch weight/volume as per specification. Stir the bulk solution for not less than XX minutes and record stirring speed, mixing time, temperature, pH, description, and final volume verification against [Calibrated Dipstick/Scale].

- Upon completion, transfer the bulk solution from the sampling valve back to the mixing vessel and record quantity transferred.

- Perform sampling of the bulk solution and send for analysis.

| Parameter | Specification |
| --- | --- |
| Bioburden [Unfiltered bulk (After bulk manufacturing stage)] |  |

- Store the bulk solution at a controlled temperature of [20–25°C] under [0.2 μm filtered nitrogen blanketing] at a pressure of not less than [X bar]. Record storage conditions and pressure details at the defined frequency.

#### First Filtration Process

Loop 3: CIP, PHT, SIP Details

- Verify the recipe parameter against the parameters available in SOP before starting the CIP cycle.

- Perform CIP and record the details and result (Complies/ Does not Comply).

- Verify the recipe parameter against the parameters available in SOP before starting the PHT cycle.

- Perform PHT and record the details (Complies/ Does not Comply).

- Ensure that vent filter integrity of filtration vessel is complying before start of SIP.

- Verify the recipe parameter against the parameters available in SOP before starting the SIP cycle.

- Perform SIP and record the details (Complies/ Does not Comply).

- Perform the line clearance activity. Record the differential pressure, temperature and % RH of the filtration room.

- Perform the product transfer (filtration) activity by filtering the solution through XX μ cartridge filter (First filter) into the filtration vessel using 0.2 μ filtered nitrogen and store under nitrogen blanketing.

- Open the vent of the filter for removal of entrapped air.

- Fill the filter completely with bulk solution and flush with approx.  XX mL bulk solution each for 2 times.

- After flushing, start the filtration by applying nitrogen pressure, record the applied pressure (NMT X Bar).

- Record the total first filtered bulk and left over solution in mixing vessel.

- Perform the sampling of bulk solution after filtration, from sampling valve and send for analysis.

| Parameter | Specification |
| --- | --- |
| Description (Visual examination) |  |
| pH |  |
| Weight per mL |  |
| Assay |  |
| Bioburden [Unfiltered bulk (After filtration)] |  |

- Store the bulk solution under 0.2μ filtered Nitrogen blanketing at pressure NLT 0.5 bar. Record the details (Frequency: every 1 hrs ± 15 min).

#### Final Filtration and Filling Process

CIP - SIP Details

- Verify the recipe parameter against the parameters available in SOP before starting the CIP cycle.

- Perform CIP and record the details and result (Complies/ Does not Comply).

- If Capsule filter is used: SIP of Loop 4 shall be performed without installing the filter in the line i.e. with empty housing.

- If Cartridge filter is used: SIP of Loop 4 shall be performed with Final filter in filter housing.

- Verify the recipe parameter against the parameters available in SOP before starting the SIP cycle.

- Perform SIP and record the details (Complies/ Does not Comply).

- After completion of SIP, connect the sterilized capsule filter assembly/ housing to the product transfer line.

- Note: During assembling verify arrow direction of Filter and fix in such a way that product flow from Filtration vessel to Filling machine.

Loop 5: CIP - SIP Details

- Verify the recipe parameter against the parameters available in SOP before starting the CIP cycle.

- Perform CIP and record the details and result (Complies/ Does not Comply).

- Verify the recipe parameter against the parameters available in SOP before starting the SIP cycle.

- Perform SIP and record the details (Complies/ Does not Comply).

Final Filtration and Filling:

- Performed the line clearance activity of vial filling line.

- Recorded the differential pressure, % RH and temperature of respective areas.

- Diffuse light source/sodium vapor lamp is turned ON.

- Ensured that UAF of vial filling, closing and capping machine is “ON” and differential pressure is within limit.

- Checked the differential pressure of mobile cart which is used for transferring the filling machine parts to filling area.

- Performed the off-line non-viable particle count (NVPC) before starting of assembling activity.

- Performed the on-line non-viable particle count (NVPC). If red light is blinking on online non-viable particle counter display due to particles above limit then stop aseptic assembling setup, aseptic activity and recorded activity details.

- Performed viable monitoring from startup of aseptic set up of vial filling machine parts.

- The vial filling, closing and capping machine is assembled. The depyrogenated vials, sterilized stoppers and seals are transferred to the respective bowl. Alarm test was performed (pre and post commencement of filling) and rejection during alarm test was recorded.

- The final filtration is carried under pressure through 0.2µ filtered Carbon dioxide (NMT X bar) and allow the solution to pass from final filter to buffer tank.

- For XX L Batch Size, the filter is filled completely with bulk solution and two filter flushes of XXX mL each are collected and discarded.

- For XXX L Batch Size, the filter is filled completely with bulk solution and 1st and 2nd filter flushes of XXX mL each are collected and discarded, and the 3rd and 4th filter flushed of XXX mL each are collected for analysis.

- The filling needles are primed for removal of entrapped air. Hold the bulk solution in Pharma XX tubing for minimum XX minute in static condition. Discard the solution available in tubing by discarding NLT XX vials (Approx. XX.XXX g) from filling machine after completion of hold before start of priming.

- The Vial filling, closing and capping machine recipe is checked for set parameters and dose setting is performed. Rejection during dose setting is recorded.

- The Vial filling, closing and capping machine are set to get the desired standard parameters mention below.

| Parameter | Set Value |
| --- | --- |
| Fill Volume |  |
| Speed |  |

- Head space oxygen content (Limit: NMT X %) after dose set up is checked and recorded.

- The Vial filling, closing and capping machine is set for operation, the machine speed, machine running time, number of good filled units, rejected units (Empty vial with rubber stopper and seal, Empty vial without rubber stopper and seal, Empty vial with rubber stopper and without seal, filled vial without rubber stopper and seal, filled vial with rubber stopper and without seal, Filled vial with rubber stopper and seal and broken vial) during filling and sealing as per predefined rejection criteria are recorded.

- The fill weight/vial, stoppering quality, sealing quality and clarity of vials are checked and recorded by manufacturing team startup of activity, every XX minutes ± X minutes and at end of the batch and by QA team during start up activity and every X hour ± XX minutes or twice in a batch whichever is more.

| Parameter | Set Value |
| --- | --- |
| Fill Volume | Target fill weight: XX g Alert limit (target fill volume ± XX%): XXg to XXg Action limit (target fill volume ± XX%): XXg to XX g |
| Stoppering Quality |  |
| Sealing Quality |  |
| Clarity of Vials |  |

- Check and record the headspace oxygen content on two consecutive filled units during initial, middle, and end stages of filling. (Acceptance criterion: NMT X%)

- Perform leak testing of sealed units by the manufacturing team at start, middle, and end of the process, and by the QA team once per batch on two consecutive filled units.

- Verify post-use integrity of the final sterilizing-grade 0.22 μm hydrophilic PVDF capsule filter. (Acceptance criterion: NLT [XXXX mbar] at [25 ± 5°C])

#### Vial Collection

- Perform line clearance activity of vial collection area.

- Collect the filled product as per SOP.

- Collect sample from sealing head and perform leak testing of collected vials at start, middle and end of Capping operation by manufacturing team and once in batch by QA team.

<Add the lyophilization parameters and procedure if the product is lyophilized. Refer to the details below for reference. >

#### Final Filtration, Vial Filling and Half Stoppering

- Performed the line clearance activity.

- Recorded the differential pressure, % RH, and temperature of the respective areas.

- Upon completion of the first filtration, the semi-auto cycle is initiated until product transfer to the [Buffer Vessel]. After first filtration, the filtered bulk solution is stirred for [XX–XX minutes] before the filling activity is started & recorded the stirring speed (Limit: [XXX ± XX RPM]) and stirring time (Limit: [XX–XX min]).

- The solution is transferred to the [Buffer Vessel] using 0.2 μm filtered nitrogen at applied pressure [X–X bar].

- Performed the line clearance activity of the [Vial Filling Line].

- Performed the on-line non-viable particle count (NVPC).

- The sterilized filling machine parts are transferred to the [Filling Machine LAF] using [Mobile LAF Unit]. Start assembly at least [XX minutes] after start of the NVPC system. The filling machine assembly is performed as per SOP.

- Ensure that the final filtration is started from [Filtration Vessel] to [Buffer Vessel] and the buffer vessel is filled with product.

- Performed the initial machine setup and recorded the quantity of vials used for machine setup.

- Perform priming to remove air from the product filling line.

- Perform the volume setting using 100% manual IPC.

- The vial filling machine is set for the required machine speed.

| Parameter | Set Value |
| --- | --- |
| Fill Volume | Targeted filled volume to XX ml/XX gm (Limit: XX to XX ml) Lower Limit (gm)= XX *Weight per mL Upper Limit (gm)= XX *Weight per mL |
| Machine Speed |  |
| Pre-gassing Pressure |  |
| Post-gassing Pressure |  |
| Filling Time |  |

- Discontinue 100% manual IPC and initiate the online/manual IPC at a frequency of [XX ± XX minutes].

- Begin the production operation of the filling machine. In case of a breakdown or a prolonged break exceeding [XX minutes], IPC is performed immediately upon resumption of filling operations.

- At the end of the filling, when the filling machine displays the "[Alarm Name]" alarm, the "[Alarm Name]" interlock is deactivated, and the filling machine continues to run to empty out the [name of vessel/tank]. IPC is performed at least [XX] times until the end of the filling.

- The rejected units (Empty vial, unstopped filled vial, filled vial, rubber stopper, leftover empty vials and leftover rubber stoppers) during filling are recorded.

- The filling line interventions are performed.

#### Lyophilization

- CIP, SIP and VLT of Lyophilizer is performed and recorded the start time and end time in BMR.

- Load vials on Shelves through auto loading and unloading system (ALUS).

- The Lyophilizer recipe:

| Lyophilization Process |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Step | Step Name | Type | Temp. (°C) | Vacuum (μbar) | Time |  |
|  |  |  |  |  | HH | MM |
| 1. | Loading |  |  |  |  |  |
| 2 | Freezing |  |  |  |  |  |
| 3. | Freezing |  |  |  |  |  |
| 4. | Freezing |  |  |  |  |  |
| 5. | Freezing |  |  |  |  |  |
| 6. | Evacuation |  |  |  |  |  |
| 7. | Drying |  |  |  |  |  |
| 8. | Drying |  |  |  |  |  |
| 9. | Drying |  |  |  |  |  |
| 10. | Drying |  |  |  |  |  |
| 11. | Drying |  |  |  |  |  |
| 12. | Drying |  |  |  |  |  |
| 13. | Drying |  |  |  |  |  |
| 14. | Drying |  |  |  |  |  |
| 15. | Drying |  |  |  |  |  |
| 16. | Drying |  |  |  |  |  |
| 17. | Drying |  |  |  |  |  |
| 18. | Drying |  |  |  |  |  |
| 19. | Drying |  |  |  |  |  |
| 20. | Stoppering |  |  |  |  |  |
| 21. | Aeration |  |  |  |  |  |
| 22. | Storage |  |  |  |  |  |
| 23. | Unloading |  |  |  |  |  |
| Average product temp. Temperature± X℃ |  |  |  |  |  |  |
| Vacuum Specification: ± XX μ bar |  |  |  |  |  |  |
| Condenser Temp. Specification: ≤ - XXC |  |  |  |  |  |  |

#### LYO Loading Activity

- The online NVPC must be started at least XX minutes before initiating vial loading.

- Select and download the appropriate recipe in the ALUS HMI.

- Start loading half-stoppered vials into the lyophilizer.

- If the number of filled vials is insufficient to complete a row or shelf as per the defined load pattern, complete the row/shelf using empty half-stoppered vials.

- Record the end time of half-stoppered vial loading into the lyophilizer.

- Record any rejections during loading.

- Record the shelf loading time, shelf loading completion time, and the total number of vials loaded per shelf.

#### LYO Unloading Activity

- Ensure the online NVPC counter has been started.

- Begin unloading of fully stoppered vials from the lyophilizer.

- The online NVPC must be started at least XX minutes prior to the start of vial unloading.

- Record the end time of stoppered vial unloading from the lyophilizer.

- Record the total quantity of vials unloaded.

- Record the shelf unloading start time, shelf unloading completion time, and the number of vials unloaded per shelf.

#### Capping Activity

- Select and download the appropriate recipe in the capping machine HMI as per the SOP.

- Perform capping machine assembly as per defined procedures.

- Enter batch details in the machine HMI and start the batch.

- Select the XX mL vial size recipe as per the applicable SOP.

- Set the machine speedx1 (XX–XXX vials/minute).

- Perform the initial machine setup as per SOP and ensure all counting counters are functioning properly.

- Record the following during setup: Start Time, End Time, Set Speed, Number of vials discarded during setup

- Conduct a counter challenge test post initial setup to ensure alignment between input counter, good counter, and rejection counter.

- If a mismatch is observed, perform reconciliation based on the infeed counter.

- Start the capping operation.

- Record the end time of capping operation upon completion.

- Perform in-process checks during capping for: Improper crimping, Loose flip-off, Dented seal, Missing flip-off, Flip-off color variation, Any other visible defects. (Frequency: At the start and end of the operation, and at every XX ± X minutes during the operation.)

#### Vial Collection & Automatic Leak test

- Perform line clearance activity of vial collection area.

- Collect sample from sealing head and perform leak testing of collected vials at start, middle and end of Capping operation.

- Performed the line clearance activity for Automatic leak test.

- Automatic leak test is performed on each & every collected vial as per respective SOP.

- The number of good and rejected vials are recorded. The rejected vials are re-leak tested in machine as per respective SOP and the number of good and rejected vials are recorded.

<Note: If the product is terminally sterilized, then the below point is applicable.>

#### Terminal Sterilization

- Perform line clearance activity of terminal sterilization area.

- Collect the filled vials and subject for terminal sterilization.

Instruction:

At the time of loading of units for terminal sterilization, ensure that all units are placed in upright position and arranged in Honeycomb pattern.

In case of loose tray/s, Place filler (like Teflon) or support (like acrylic sheet) in the tray before loading the tray for terminal sterilization.

- Load the filled vials in steam sterilizer chamber as per validated load pattern and perform the terminal sterilization [F0 value: XX minutes].

- After terminal sterilization completion, unload all sterilized vials in unloading area and update label on each tray.

- Record the quantity of vials transferred for visual inspection.

#### Visual Inspection

- Perform line clearance activity and record the temperature (XX°C – XX°C) of visual inspection area.

- Perform visual inspection of each vial and record the details.

- Record the total vials taken for 1st 100% visual inspection, total rejected vials and total good vials after 1st 100% visual inspection.

- Perform in-process checking of visual inspected vials by sampling randomly NLT XX vials during visual inspection at start up and every XX Minutes ± XX minutes by manufacturing team and record the good vials and rejected vials.

| Critical Rejections (NMT 1%) |  |
| --- | --- |
| Major Rejection (NMT 3%) |  |
| Minor Rejection (NMT 5%) |  |
| Total Rejections: NMT 5% |  |

- Carry out 2nd 100% visual inspection operation on the vials collected after 1st visual inspection and record total vials taken for 2nd 100% visual inspection, total rejected vials and total good vials after 2nd 100% visual inspection.

- Perform in-process checking of visual inspected vials by sampling randomly NLT XX vials during visual inspection at start up and every XX Minutes ± XX minutes by manufacturing team and record the good vials and rejected vials.

- Record the total number of vials inspected, total number of rejected vials (1st inspection, 2nd inspection and during in-process) and total number of good, inspected vials.

- Perform AQL (Acceptance Quality Limit) after completion of visual inspection and record the number of vials rejected during AQL.

- Collect the rejected units (if any) in duly labeled container for destruction. Remove the flip part of seal from vial, once it is rejected.

- Transfer the good visually inspected vials for packing.

#### Labeling and Packaging

- Perform line clearance activity for issuance of secondary packaging material.

- Issue the secondary packaging material as per requirement and record the information.

- Check the product description before start of packing activity.

- Before start of packaging, instruct all the line staff for pack style and operations.

- Check the coding details of the sticker label, carton and shipper label and record to approve the labels.

- Record temperature (Limit: XX ± X°C) of packaging room before start of packing activity. Perform the line clearance activity of vial packing area and equipment.

- Perform the camera challenge test Camera Challenge test performed for verification of no label detection, missing batch coding details, Smudge batch coding detail, missing barcode on label, Smudge barcode on label, Verification of cap sensor and record the observations. Frequency of challenge test before start of the batch/ operation, End of batch/ operation, Break down maintenance/ power failure.

- Label the received goods and check for correctness and further pack in carton as per pack style.

- Perform the de-labeling and re-labeling activity.

- Record the details of each instruction followed during labeling and packaging.

- Perform the in-process checks as per predefined parameters on NLT XX units at start up, after every XX minutes ± X minutes and at the end of packaging (by manufacturing team); and start of the batch, after every X hours ± XX minute or twice in a batch, whichever is more (by QA).

- Perform AQL of packed goods.

- Perform yield reconciliation for quantity packed as well as for each packaging materials.

### Stage-wise Proposed Yield

| Stage | Proposed Yield |
| --- | --- |
| Unfiltered Bulk Solution |  |
| First filtered bulk |  |
| Filling Stage |  |
| Lyophilization stage (If applicable) |  |
| Capping Stage |  |
| Collection Stage |  |
| Filled Units |  |
| Terminal sterilization stage (If applicable) |  |
| Visual Inspection Stage |  |
| Packing Stage |  |
| Batch Yield |  |

<Note: Please ensure that each process step and acceptance criterion matches the commercial batch manufacturing and packaging record.>

### Holding Period Specifications

- Holding period specifications proposed for [Product Name] are provided in 32p34-control-critical-steps. The holding period specifications have been validated based on the hold time studies performed on [Product Name]. The hold time validation studies have been summarized under 32p35-process-validation.

## Manufacturing Process Flow Chart

<Please include a manufacturing flow chart that lists all process steps, incoming materials, and in-process controls for each step, ensuring consistency with Section 3.2.P.3.4>

## Master Batch Manufacturing and Packaging Records

A comparative overview of the exhibit and proposed commercial batch size is tabulated below:

Table 3. Executed Vs. Proposed Commercial Batch Size Overview

| Strength/ Fill Volume | Executed Batch Size | Proposed Commercial Batch Size | Scaled-Up Factor |
| --- | --- | --- | --- |

The above tabulated comparison confirms that the scale-up factor for the proposed commercial batch sizes is up to 10x with respect to the exhibit batch size.

The proposed commercial batch manufacturing record, batch packaging record and a comparison summary between the exhibit and commercial batch records have been included in this module and the references for the same have been tabulated below.

Table 4. Proposed Commercial Batch Record Reference

| Product Name | Batch Manufacturing Record | Batch Packaging Record |
| --- | --- | --- |
|  | BMR number | BPR number |
|  | BMR Comparison (Rationale) |  |

## Sterilization/Depyrogenation

[Product Name] is a Clear, colorless solution for intravenous administration. The proposed drug product is supplied in proposed container closure system. The proposed drug product is manufactured by “Terminal Sterilization/ Aseptic sterilization” approach by moist heat.

<Note: If the product is manufactured using the aseptic sterilization approach, include the same points except for terminal sterilization. In addition, add details on equipment sterilization (SIP) and component sterilization (rubber stopper sterilization). Please provide the information under the topics mentioned below, based on the company-specific equipment qualification reports and protocols.

Equipment Sterilization Process and critical Parameters

Steam Sterilizer Details

Steam Sterilizer Cycle Parameters

Methods and Controls to Monitor Production Cycles

Steam Sterilizer Load Patterns

Load description

Requalification or Revalidation

### Terminal Sterilization (moist heat) [if applicable]

#### Terminal Sterilization Process and Critical Parameters

- The drug product filled vials are sterilized using moist heat in a Steam Sterilizer.

- The Steam Sterilizer make, and model details are tabulated below:

Table 5. Steam Sterilizer Details

| Equipment Name | Equipment Id # | Model No. | Manufact Urer | Principle/ Used For | Loads Type | Cycle Type |
| --- | --- | --- | --- | --- | --- | --- |
| Steam Sterilizer |  |  |  |  |  |  |

- The critical control parameters for the moist heat sterilization cycle in Steam Sterilizer for each of the equipment is provided in below table.

Table 6. Steam Sterilizer Cycle Parameters (Note: Parameters are changed based on equipment and model)

| Parameters | Set Value |
| --- | --- |
| Pre Heat Temperature (°C) |  |
| Pre Heat Time (Sec) |  |
| Sterilize Temperature (°C) |  |
| Sterilize Time (Sec) |  |
| F0 Value (Min) |  |
| Slow Cool Temperature (°C) |  |
| Fast Cool Temperature (°C) |  |
| Heat Pressure Coef |  |
| Ster Pressure Coef |  |
| Cool Pressure Coef |  |
| High Temp Alarm Sv (°C) |  |
| Low Temp Alarm Sv (°C) |  |
| Steam Large Close Temp (°C) |  |
| Pid Parameters |  |
| Pid Temp 1 Set (°C) |  |
| Pid Out 1 Set (%) |  |
| Pid Temp 2 Set (°C) |  |
| Pid Out 2 Set (%) |  |
| Pid Temp 3 Set (°C) |  |
| Pid Out 3 Set (%) |  |
| Pid Temp 4 Set (°C) |  |
| Pid Out 4 Set (%) |  |

#### Methods and Controls to Monitor Production Cycles

- Sterilization cycle is controlled by inbuilt temperature probe located inside the chamber.

- During routine production cycles, the validation F0 value and set parameters are incorporated into the Terminal Sterilization’s operating program.

- The temperature and exposure time of the Terminal sterilizer for this product is based on F0 value.

- During routine production cycle, loaded quantity of drug product will be within validated maximum and minimum load pattern.

#### Steam Sterilizer Load Patterns

- The equipment Steam Sterilizer loading patterns summary for commercial production are provided in below table. The arrangement of the components within the loads has been detailed under Module 32p35-process-validation.

Table 7. Steam Sterilizer Load Details

| Sr. No. | Component | Load# | Load Type | Load Description |
| --- | --- | --- | --- | --- |
| 1 | Vial | XX | Minimum | Please Refer Table XX |
|  |  | XX | Maximum | Please Refer Table XX |

Table 8. Vials (Minimum Load): Terminal Sterilization Load Details

| Recipe | Load Pattern | No. of Trolley | No. of Layer | No. of PP tray/ layer | No. of Containers/ PP Tray | Total No. of Containers |
| --- | --- | --- | --- | --- | --- | --- |

Table 9. Vials (Maximum Load): Terminal Sterilization Load Details

| Load Pattern | No. of Trolley | No. of Layer | No. of PP tray/ layer | Total No. of PP Trays/ Trolley | No. of Container/ PP Tray | No. of Container/ Layer | Total No. of Containers/ Trolley | Total No. of Containers |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

#### Requalification or Revalidation

- Annually one run of Empty Chamber Heat Distribution (ECHD) Study shall be carried out to evaluate inherent capability of terminal sterilization to achieve temperature stability and uniformity of the empty chamber during Sterilization hold period. In case of any change in recipes, three runs of Empty Chamber Heat Distribution (ECHD) Study shall be carried out.

- Annually one run for Loaded Chamber Heat Penetration (HP) with BI Challenge study for identified loads (Maximum Load only) shall be carried out to verify that minimum lethality is achieved in the product container throughout the load inside the chamber and product contents inside containers (vials) are sterilized. In case of introduction of new load / revised load / change in recipe or as per CCF assessment, three runs shall be carried out.

### Component Depyrogenation

#### Depyrogenation Process and Critical Parameters

- The drug product containers i.e. XX ml USP Type I clear glass vials are depyrogenated using dry heat in a depyrogenation tunnel.

- The depyrogenation tunnel make, model and manufacturer details are tabulated below:

Table 5. Depyrogenation Tunnel Details

| Equipment Name | Equipment ID # | Model No. | Manufacturer | Principle/Used For |
| --- | --- | --- | --- | --- |
| Vial Depyrogenation Tunnel |  |  |  |  |

- The depyrogenation parameter of the drug product containers are tabulated below:

Table 6. Depyrogenation Parameters

| PARAMETERS | X mL Glass Vials |
| --- | --- |
|  | SET VALUE |
| heating temp. (°C)* |  |
| Belt Speed (mm/min.) |  |
| Air shutter at heating zone inlet open height (mm) |  |
| Air shutter at heating zone outlet open height (mm) |  |
| Cooling Temp. |  |
| Pre-heating Temp. |  |

#### Methods and Controls to Monitor Production Cycles

- Depyrogenation and cooling of the vials in the sealed tunnel carried out under cleanliness class. The sealed unit is divided into three zones, the first zone is preheating zone, second is heating zone and third is cooling zone.

- During routine production, conveyer belt speed & temperature is kept same as per validation.

#### Depyrogenation Load Patterns

- The XX ml USP Type I clear glass vials will be depyrogenation by continuous operation through the depyrogenation tunnel during commercial production.

#### Requalification or Revalidation

- Periodic requalification is performed on the XX ml USP Type I clear glass vials at frequency of 1 year ± X month with one run for “Loaded Chamber Heat Distribution- Penetration study”.

- In case of Major Modification in equipment, Adjustment in equipment and modification in exiting tunnel, XXX run for “Loaded Chamber Heat Distribution- Penetration study” is to be performed.

### Component Sterilization

#### Sterilization Process and Critical Parameters

- The drug product rubber stoppers and seals are sterilized using moist heat in an autoclave.

- The autoclave make, model and manufacturer details are tabulated below:

Table 10. Autoclave Details

| Equipment Name | Equipment ID # | Model No. | Manufacturer | Principle/ Used For | Loads Type | Cycle Type |
| --- | --- | --- | --- | --- | --- | --- |

- The critical control parameters for the moist heat sterilization cycle in Autoclave for each of the rubber stoppers and seals are tabulated below:

Table 11. Autoclave Cycle Parameters <Note: Parameters is changed based on equipment and model>

| Autoclave Cycle Parameter | XX MM Rubber Stopper and seals |
| --- | --- |
|  | Set Values |
| No. of pre-pulse (Nos) |  |
| Pre-vacuum (bar) |  |
| Pre-vacuum hold time (min.) |  |
| Pre-pressure (bar) |  |
| Pre-pressure Hold time (min.) |  |
| Sterile hold temp. (°C) |  |
| Control Band (°C) |  |
| Sterile hold time (min) |  |
| Sterile hold reset (°C) |  |
| Sterile hold stop count (°C) |  |
| Sterile temp. overshoot (°C) |  |
| Jacket control temp. (°C) |  |
| No. of post pulse (Nos) |  |
| Drying vacuum (bar) |  |
| Drying vacuum hold time (min.) |  |
| Drying Pressure (bar) |  |
| Drying Pressure hold time (min) |  |
| Drying Jacket temp. (°C) |  |
| Post Vacuum (bar) |  |
| Door open temp. (°C) |  |
| Print interval (Sec) |  |
| Cond bypass off temp. (°C) |  |

#### Methods and Controls to Monitor Production Cycles

- The moist heat sterilization cycle in Autoclave is controlled by temperature sensors located inside the drain and chamber.

- During validation, cycles are controlled with reference to these temperature sensors.

- During routine production cycles, the validated cycle time and set parameters are incorporated into the autoclave’s operating program.

- The sterilization hold set temperature is XXX.0°C – XXX.0°C, pressure between X.0 bar to X.X bar and sterilization hold time of the autoclave is NLT XX minutes.

#### Autoclave Load Patterns

- The rubber stopper  and seals autoclave loading pattern summary for commercial production are tabulated below. The arrangement of the components within the loads has been detailed under Module 3.2.P.3.5.

Table 12. Autoclave Load Details

| Sr. No. | Component | Load# | Load Type | Load Description |
| --- | --- | --- | --- | --- |
| 1 | XX mm rubber stopper and seals | XX | Minimum |  |
|  |  | XX | Maximum |  |

#### Requalification or Revalidation

- Annually one run of Empty Chamber Heat Distribution (ECHD) Study shall be carried out to verify the temperature stability and uniformity of the empty chamber during Sterilization hold period. In case of any change in recipes, three runs of Empty Chamber Heat Distribution (ECHD) Study shall be carried out.

- Annually one run for Loaded Chamber Heat Distribution & Heat Penetration (HDHP) with BI Challenge study for identified loads shall be carried out to verify the sterilization efficiency of identified loads using the steam sterilizer. In case of any new load or modified load, three runs or one run as specified in CCF shall be carried out.

### Equipment Sterilization [if applicable]

#### Sterilization Process and Critical Parameters of Autoclave

- The drug product Filling and Capping Load are sterilized using moist heat in an autoclave.

- The autoclave make, model and manufacturer details are tabulated below:

Table 8. Autoclave Details

| Equipment Name | Equipment ID # | Model No. | Manufacturer | Principle/ Used For |
| --- | --- | --- | --- | --- |

- The critical control parameters for the moist heat sterilization cycle in Autoclave for Filling and Capping Load is tabulated below:

Table 9. Autoclave Cycle Parameters

| PARAMETERS | Filling and Capping Load |
| --- | --- |
|  | SET VALUE |
| No. of Pre Pulse |  |
| Pre Pressure |  |
| Pre Pressure Control Band |  |
| Pre Pressure Hold Time |  |
| Sterile Hold Temperature |  |
| Control Band |  |
| Sterile Hold Time |  |
| Sterile Hold Reset |  |
| Sterile Hold Stop Count |  |
| Sterile Temperature Overshoot |  |
| Jacket Control Temperature |  |
| No. Of Post Pulse |  |
| Door Open Temperature |  |
| Print Interval |  |
| Cond Bypass off Temperature |  |
| Control Temp. |  |

#### Methods and Controls to Monitor Production Cycles

- The moist heat sterilization cycle in Autoclave is controlled by temperature sensors located inside the drain and chamber.

- During validation, cycles are controlled with reference to these temperature sensors.

- During routine production cycles, the validated cycle time and set parameters are incorporated into the autoclave’s operating program.

- The sterilization hold set temperature is XXX.0°C – XXX.0°C, pressure between XX bar to XX bar and sterilization hold time of the autoclave is NLT XX minutes.

#### Autoclave Load Patterns

- The Filling and Capping Load summary for commercial production are tabulated below. The arrangement of the components within the loads has been detailed under Module 32p35-process-validation.

Table 10. Autoclave Load Details

| Sr. No. | Component | Load Type | Load Description |
| --- | --- | --- | --- |
| 1 | Filling and Capping Load | Minimum |  |
|  |  | Maximum |  |

#### Requalification or Revalidation

- Periodic requalification is performed on maximum load of the Filling and Capping Load at frequency of 1 year ± X month with one run for “Loaded Chamber Heat Distribution- Penetration study”.

### Equipment Sterilization (Filtration Vessel) [if applicable]

#### Sterilization Process and Critical Parameters of Filtration Vessel

- The Sterilization in Place performed for Filtration vessel.

- The Filtration vessel make, model and manufacturer details are tabulated below:

Table 11. Lyophilizer Details

| Equipment Name | Equipment ID # | Model No. | Manufacturer | Principle/ Used For |
| --- | --- | --- | --- | --- |

- The critical control parameters for the sterilization in place for the Filtration vessel are tabulated below:

Table 12. Filtration vessel SIP Cycle Parameters

| Parameter | Set Value |
| --- | --- |
|  | Filtration Vessel |
| Heating Temperature (°C) |  |
| Heating Pressure (Bar) |  |
| Heating Hold Timer (Min) |  |
| Air Drying Cycle Start Pressure (Bar) |  |
| Sterilization Time (Min) |  |
| Sterilization Temperature (°C) |  |
| Sterilization Stop Time Temperature (°C) |  |
| Sterilization Reset Time Temperature (°C) |  |
| Air Drying End Temperature (°C) |  |
| Positive Pressure (Bar) |  |
| Air Drying Valve Open Pressure (Bar) |  |
| Air Drying Valve Close Pressure (Bar) |  |
| Positive Pressure Selection |  |
| Jacket Cooling Temperature (°C) |  |

#### Methods and Controls to Monitor Production Cycles

- The Sterilization-In-Place (SIP) cycle in Filtration vessel is controlled by temperature sensors placed across the chamber.

- SIP cycle is monitored using calibrated thermocouples and an external data logger to ensure uniform heat distribution across all critical points.

- F0 value ≥ XX minutes must be achieved at all locations, with a minimum hold temperature of XXX°C.

#### Filtration Vessel Load Patterns

- The SIP of the Filtration vessel is performed during commercial production. The arrangement of the components within the loads has been detailed under Module 32p35-process-validation.

#### Requalification or Revalidation

A periodic requalification is performed 1 Year ± X Month, wherein one “sterilization in place” run is to be performed.

### Equipment Sterilization (Lyophilizer)

#### Sterilization Process and Critical Parameters of Lyophilizer

- The Sterilization in Place performed for Lyophilizer.

- The Lyophilizers make, model and manufacturer details are tabulated below:

Table 13. Lyophilizer Details

| Equipment Name | Equipment ID # | Model No. | Manufacturer | Principle/ Used For |
| --- | --- | --- | --- | --- |
| Lyophilizer |  |  |  |  |

- The critical control parameters for the sterilization cycle for the Lyophilizer, are tabulated below:

Table 14. Lyophilizer SIP Cycle Parameters

| Parameters | Set Value |
| --- | --- |
|  | Lyophilizer |
| Sterilization Temperature |  |
| Sterilization Time |  |
| Purge Number |  |
| Control Pressure |  |
| Pre-Evacuation Pressure |  |
| Steam Discharge Pressure |  |
| Filter Drying Time |  |
| System Drying Time |  |
| Condenser Cooling Time |  |
| Shelf Cooling Time |  |
| Jacket Cooling Time |  |
| Time To Reach Set Point |  |
| Total Time After Set Point Reached |  |
| Time Below Set Point Once Reached |  |

#### Methods and Controls to Monitor Production Cycles

- The Sterilization-In-Place (SIP) cycle in Lyophilizer is controlled by temperature sensors placed across the chamber.

- F0 value ≥ XX minutes must be achieved at all locations, with a minimum hold temperature of XXX°C.

#### Lyophlizer Load Patterns

- The SIP of the lyophilizer is performed during commercial production. The arrangement of the components within the loads has been detailed under Module 32p35-process-validation.

#### Requalification or Revalidation

A periodic requalification is performed X Year ± X Month, wherein one “sterilization in place” run is to be performed.

### Environmental Monitoring

<Note: Please add the environmental monitoring information from the company-specific SOP under the topics mentioned below.>

#### Microbial Monitoring

<Add the Procedure and frequency.>

Microbiological Monitoring of Environment in Production Area

There are two methods of environmental monitoring described in this procedure

  - Non-Viable Monitoring

  - Viable Monitoring

A. Non-Viable Air Particulate count:

< Add the procedure, acceptance limits, and monitoring frequency for non-viable air particulate count.>

B. Viable Particulate Monitoring

Bio Burdon Monitoring Program for WFI (Water for Injection)

<Add the procedure and frequency.>

- Product Component Bioburden

The bioburden of the bulk solution of drug product is monitored as per the current version of SOP XXXX.

Bioburden samples are collected after manufacturing of bulk, after 1st filtration and final filtration and tested as per in process specification. The bio burden results of exhibit batches are summarized in Module 3.2.P.3.4.

| Stage | Bio burden Limit |
| --- | --- |
| In Process Product Specification |  |

- Investigation in case of Excursion of Limit

When an environment monitoring excursion i.e., OAL/OAC/OOS/OOT or AT results are observed in manufacturing area, the analyst shall log in excursion details in respective format and notify the Microbiology Head or designee, Manufacturing/Warehouse Head or designee, Engineering Head or designee and Quality Assurance Head or designee as per respective exhibit and action shall be taken as defined in SOP:XXXXXX.

<Note: Investigation procedures for any excursion beyond the limits should be as per the company-specific SOP, and the SOP should be provided in the submission>

< The Environmental Monitoring SOP also needs to be provided in the submission.>

## Reprocessing Statement

Reprocessing statements are attached in this section for both, ANDA applicant (Applicant name) and drug product manufacturer (Drug product Manufacturer name) as mentioned below.

1. Applicant – Applicant name’s Reprocessing Statement

2. Drug Product Manufacturer – Name of Manufacturer ’s Reprocessing Statement