Table Of Contents

1	Dissolution Development Report	2

1.1	Specification:	2

1.2	Justification:	2

1.2.1	Justification of Dissolution Conditions	2

1.2.2	Justification for addition of Tier II:	3

1.3	Justification of Specification	4

2	discrimination power of dissolution method	4

2.1	Level of Gelucire:	4

2.1.1	Conclusion:	5

2.2	Level of Span 80:	6

2.2.1	Conclusion:	7

3	Overall Conclusion	7

List of In-Text Tables

Table 1.	Summary of Dissolution Test Conditions	3

Table 2: Drug Name Capsules, xx mg: Evaluation of Effect of Gelucire Level on Drug Product Dissolution	5

Evaluation of Effect of Gelucire Level on Drug Product Dissolution	5

Table 3: Drug Name Capsules, XX mg: Evaluation of Effect of Gelucire Level on Drug Product Dissolution	6

List of In-Text Figures

Figure 1 	Dissolution data for Drug Name with different level of Gelucire	5

Figure 2 	Dissolution data for Drug Name with different level of Span 80	6

# Dissolution Development Report

## Specification:

| Time Point | % Drug Release |
| --- | --- |
| 90 minutes | NLT 70% (Q) |

## Justification:

The development of Proposed Product Name at Company Name was guided by the dosage form design and the drug delivery system of the Reference Listed Drug (RLD) Product name®. The brand product Product name® is formulated as an immediate release semi-solid 2 piece hard gelatin capsule dosage from. The product is indicated for the <Add indication>.

The formulation development of Company Name Pharmaceutical’s generic version was designed as an active suspension filled into hard gelatin capsules. All strengths of Drug Name capsules share the same formulation component which is filled into capsules proportionally to achieve the capsule strength.

There are six different dissolution test listed in USP monograph for Proposed Product Name, Company Name adopt Test-3 dissolution method from USP Monograph for Proposed Product Name as RLD (Product name®) for routine QC release purpose. The analytical procedure for the determination of dissolution of Proposed Product Name, is reverse phase high pressure liquid chromatographic method.

### Justification of Dissolution Conditions

The dissolution conditions are referred from the USP Monograph for Proposed Product Name and consistent with reference listed drug (RLD) Product name®. The analytical procedure for determination of dissolution of Proposed Product Name is in line with USP (Test-3) and modified according to USP General chapter <621> for it’s intended use and validated under actual lab conditions. In addition, Company Name also proposed Tier-II dissolution method for stability testing. Please refer to Module 3.2.P.5.2 for dissolution test procedure and Module 3.2.P.5.3 for method validation report. Please refer to Table 1 for the summary of dissolution conditions.

| Table 1. Summary of Dissolution Test Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Attributes | Tier-I (For Release) | Tier-II (For Stability Testing) | Justification |
| Medium |  |  | Consistent with USP dissolution Test-3 |
| Volume | 900 mL |  |  |
| Apparatus | USP Apparatus II (Paddle) with Spiral sinker |  |  |
| Speed | 75 rpm |  |  |
| Temperature | 37°C ± 0.5°C |  | Consistent with the USP General Chapter <711> |
| Tolerance | NLT 70% (Q) at 90 minutes |  | Refer to Section 1.5 for justification |

### Justification for addition of Tier II:

<Please add proper justification for addition of Tire -II testing for dissolution. Here for example text is provided. Please add product specific justification.>

Company Name would like to inform the Agency that according to dissolution Test-3 of USP Monograph for Proposed Product Name, Tier-I dissolution conditions are recommended for finished product release. However, Company Name has also proposed Tier II dissolution medium which is common for Hard gelatin capsule dosage form. Analytical procedure allows to perform the dissolution test with Tier-II dissolution parameters. Overall composition of dissolution medium is similar in the Tier-I and Tier II, except pretreatment step included for stability testing to avoid denature of Pancreatin in presence of the surfactant (Name) as recommended in USP<711>.

During stability testing for dissolution analysis in Tier-1 it had been observed that due to cross linking of the gelatin shell, rupture of the capsule shell is delayed. Dissolution medium uses 50g/L of pancreatin, however, to improve effectiveness of pancreatin and to avoid denature of Pancreatin in presence of the surfactant (cetrimide) pretreatment step included for stability testing.

## Justification of Specification

Dissolution conditions and sampling point and specification are consistent to the dissolution test-3 of current USP monograph for Proposed Product Name. Please refer Module 5.3.1.3 for dissolution data.

# discrimination power of dissolution method

Company Name has attempted a few trials to check the discrimination power of dissolution method using Tier-1. Company Name has manufactured two R&D batches with different levels of excipients composition.

The discriminating ability of the method has been adequately investigated / demonstrated with followings:

- Level of Surfactant

- Level of Span 80

## Level of Gelucire:

To understand the discriminating ability of the dissolution method with different levels of Gelucire, Company Name conducted dissolution study with Tier-1 method (USP Test-3).

R&D batch was manufactured using different levels of Gelucire in the formulation. The results for the different levels of Gelucire are summarized in Table 2. The batch #xxxxxx and batch# xxxxxxxx were manufactured with about 48.77% & 68.29% Gelucire respectively while process engineering batch# xxxxxxxx of final formulation contains about 58.52% Gelucire with respect to Active suspension.

Company Name evaluated the dissolution data with different levels of Gelucire using Tier-I method as follows. The results for dissolution are provided in the following table.

| Dissolution Method Parameter (Tier-I) |  |
| --- | --- |
| Medium | Buffer at pH 8.0 containing 50 mg/L of pancreatin and 5gm/L of Cetrimide (0.5%) |
| Volume | 900 mL |
| Apparatus | USP Apparatus II (Paddle) with Spiral Sinker |
| Speed | 75 rpm |
| Temperature | 37°C ± 0.5°C |
| Time Point (Minutes) | 15, 30, 45, 60 and 90 Minutes |

Table 2: Proposed Product Name, xx mg: Evaluation of Effect of Gelucire Level on Drug Product Dissolution

| Evaluation of Effect of Gelucire Level on Drug Product Dissolution |  |  |  |
| --- | --- | --- | --- |
| Time Point (Minutes) | #xxxxxxx | #xxxxxxx | #xxxxxxx |
|  | (About 48.77% Gelucire) | (About 58.52% Gelucire) | (About 68.29% Gelucire) |
| 0 | 0 | 0 | 0 |
| 15 | 14 | 8 | 8 |
| 30 | 41 | 36 | 29 |
| 45 | 70 | 66 | 52 |
| 60 | 96 | 87 | 74 |
| 90 | 109 | 100 | 101 |
| Book/Page | AR-BR-XXX-XX/XXX | AR-BR-XXX-XX/XXX | AR-BR-XXX-XX/XXX |

Figure 1 	Dissolution data for Drug Name with different level of Gelucire

### Conclusion:

The dissolution profile for Drug Name drug release clearly demonstrates discrimination for different levels of Gelucire in the formulation. The proposed dissolution method with different levels of Gelucire proves discriminating ability. Method showed adequate discrimination and met the intended purpose of discrimination.

## Level of Span 80:

To understand the discriminating ability of the dissolution method with different levels of Span 80, Company Name conducted dissolution study with Tier-1 method (USP Test-3). R&D batch was manufactured using different levels of Span 80 in the formulation. The results for the different levels of Span 80 are summarized in Table 3. The batch # xxxxxxx and batch# xxxxxxx were manufactured with about 0.0 % & 8.30% Span 80 respectively while process engineering batch # xxxxxxx of final formulation contains about 4.88% Span 80 with respect to Active suspension.

Company Name evaluated the dissolution data with different levels of Span 80 using Tier-I method as follows. The results for dissolution are provided in the following table.

Table 3: Proposed Product Name, XX mg: Evaluation of Effect of Span 80 Level Level on Drug Product Dissolution

| Evaluation of Effect of Span 80 Level on Drug Product Dissolution |  |  |  |
| --- | --- | --- | --- |
| Time Point (Minutes) | #xxxxxxx | #xxxxxxx | #xxxxxxx |
|  | (0% Span 80) | (About 4.88% Span 80) | (About 8.30% Span 80) |
| 0 | 0 | 0 | 0 |
| 15 | 6 | 8 | 1 |
| 30 | 26 | 36 | 8 |
| 45 | 48 | 66 | 19 |
| 60 | 68 | 87 | 31 |
| 90 | 81 | 100 | 47 |
| 120 | 83 | 101 | 65 |
| Book/Page | AR-BR-XXX-XX/XXX | AR-BR-XXX-XX/XXX | AR-BR-XXX-XX/XXX |

Figure 2 	Dissolution data for Drug Name with different level of Span 80

### Conclusion:

The dissolution profile for Drug Name drug release clearly demonstrates discrimination for different level of Span 80 in the formulation. The proposed dissolution method with different levels of Span 80 have discriminating ability. Method showed adequate discrimination and met the intended purpose of discrimination.

# Overall Conclusion

Based on all the above assessments, Company Name has drawn following conclusions:

Company Name’s current dissolution conditions are consistent with Dissolution Test-3 of USP Monograph for Drug Name Capsules.

Company Name’s current dissolution conditions shows discrimination power with respect to critical formulation variables and material aspects.

The dissolution results for finished product release and stability analysis of Proposed Product Name, meet the specification limit either in Tier 1 or in Tier 2 conditions using current method. The chromatographic method used for measurement of dissolved content of Proposed Product Name is specific, linear, precise and accurate and consistent with test-3 of USP Monograph of Proposed Product Name.

Company Name believes that the proposed regulatory specification of NLT 70% (Q) in 90 minutes for Drug Name is in line with current USP Monograph of Proposed Product Name is acceptable and indicating that the method is suitable to evaluate the critical quality attributes of future batches.